Characteristics of transmission lines in the transmission scheme of the Republic of Tajikistan

| № | Distance | U, [kV] | Brand and conductor cross-section | L, [km] | Number of circuits | Number of distribution phases | I max, [A] |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dushanbe -Sugd | 500 | AC400 | 213 | 1 | 3 | 2000 |
|  | Dushanbe - Regar | 500 | AC400 | 50 | 1 | 3 | 2000 |
|  | Nurek HPP - Regar | 500 | AC400 | 110.9 | 1 | 3 | 2000 |
|  | Nurek HPP - Regar | 500 | AC400 | 114.9 | 1 | 3 | 2000 |
|  | DTPP №2 - Dushanbe | 220 | АС-400 | 8,3 | 2 | 1 | 835 |
|  | Dushanbe- Novaya | 220 | AC300+AC400 | 4.2+6.8 | 1 | 1 | 600 |
|  | Dushanbe - Novaya | 220 | AC300+AC400 | 0.3+10.6 | 1 | 1 | 600 |
|  | Regar - Novaya | 220 | AC300 | 47.5 | 1 | 1 | 600 |
|  | Dushanbe – Orjabad 2 | 220 | AC300+AC400 | 19.7+6.8 | 1 | 1 | 600 |
|  | Orjabad 2 – T Toj Chaina | 220 | АС-300 | 32,2 | 1 | 1 | 600 |
|  | T Toj Chaina– Nurek T1 | 220 | AC300 | 15 | 1 | 1 | 600 |
|  | Nurek – Nurek T1 | 220 | AC300 | 0.5 | 1 | 1 | 690 |
|  | Nurek HPP – Nurek T1 | 220 | AC300 | 2.5 | 1 | 1 | 690 |
|  | Nurek HPP – Nurek T2 | 220 | AC300 | 2.5 | 1 | 1 | 690 |
|  | Nurek – Nurek T2 | 220 | AC300 | 0.5 | 1 | 1 | 690 |
|  | Yavan TPP– Nurek T2 | 220 | AC300 | 44.7 | 1 | 1 | 690 |
|  | Nurek – Rogun | 220 | AC300 | 61.3 | 1 | 1 | 600 |
|  | Nurek – Sebiston | 220 | AC300 | 25 | 1 | 1 | 600 |
|  | Sebiston - Lolazor | 220 | AC300 | 18.1 | 1 | 1 | 600 |
|  | Lolazor – Hatlon | 220 | AC400 | 56.3 | 2 | 1 | 600 |
|  | Lolazor – Sangtuda HPP-1 | 220 | AC400 | 34.6 | 2 | 1 | 600 |
|  | Lolazor – Baipaza HPP | 220 | AC300 | 36 | 1 | 1 | 600 |
|  | Sangtuda HPP-1 – Sangtuda HPP-2 | 220 | AC400 | 13 | 1 | 1 | 600 |
|  | Golovnaya HPP – Sangtuda HPP-2 | 220 | AC400 | 12 | 1 | 1 | 600 |
|  | Golovnaya HPP – Pryadilnaya | 220 | AC400 | 16.2 | 1 | 1 | 400 |
|  | Golovnaya HPP – Kolhozobod | 220 | AC300 | 49 | 1 | 1 | 600 |
|  | Rumi– Pryadilnaya | 220 | AC300 | 75.6 | 1 | 1 | 600 |
|  | Geran – Kolhozobod | 220 | AC240 | 30 | 1 | 1 | 300 |
|  | Geran - Rumi | 220 | АС-300 | 78 | 1 | 1 | 600 |
|  | Dushanbe - Shahrinav | 220 | AC300+AC400 | 34,6+10.6 | 1 | 1 | 600 |
|  | Dushanbe – Jangal | 220 | AC300 | 28 | 1 | 1 | 600 |
|  | Yavan TPP– Jangal | 220 | AC300 | 31.8 | 1 | 1 | 600 |
|  | Baipaza HPP – Jangal | 220 | AC300 | 53.4 | 1 | 1 | 690 |
|  | Baipaza HPP – Yavan T1 | 220 | AC300 | 56 | 1 | 1 | 600 |
|  | Yavan – Yavan T1 | 220 | AC300 | 1.4 | 1 | 1 | 600 |
|  | Baipaza HPP – Yavan T1 | 220 | AC300 | 8.7 | 1 | 1 | 600 |
|  | Baipaza HPP – Yavan T2 | 220 | AC300 | 9.5 | 1 | 1 | 690 |
|  | Yavan – Yavan T2 | 220 | AC300 | 31 | 1 | 1 | 690 |
|  | Yavan TPP– Yavan T2 | 220 | AC300 | 1.5 | 1 | 1 | 690 |
|  | Regar - Ravshan | 220 | АС-300 | 5,6 | 2 | 1 | 690 |
|  | Sugd– Shahriston T | 220 | AC400 | 90 | 1 | 1 | 600 |
|  | Aini – Shahriston T | 220 | AC400 | 28 | 1 | 1 | 600 |
|  | Sugd –Khujand | 220 | AC300+AC400 | 42.2+12 | 1 | 1 | 600 |
|  | Sugd –Kairakum HPP | 220 | AC300 | 65 | 1 | 1 | 600 |
|  | Kairakum HPP –Khujand | 220 | AC300 | 21 | 1 | 1 | 600 |
|  | Kairakum HPP - Asht | 220 | АС-300 | 74 | 1 | 1 | 600 |
|  | Kairakum HPP –Kanibadam | 220 | AC300 | 66.7 | 1 | 1 | 600 |
|  | Kanibadam –Aigul Tash | 220 | AC400 | 53.2 | 1 | 1 | 600 |
|  | Sugd –Buston | 220 | AC300+AC400 | 20+12 | 1 | 1 | 600 |
|  | Buston – Kairakum HPP | 220 | AC300 | 62 | 1 | 1 | 600 |
|  | Sugd– KNS2 | 220 | AC400 | 8 | 2 | 1 | 690 |
|  | Yzlovaya – KNS2 | 220 | AC300+AC400 | 5+1 | 2 | 1 | 690 |
|  | KNS1– KNS 2 | 220 | AC300 | 4.9 | 2 | 1 | 690 |
|  | Yzlovaya - Sirdarya | 220 | AC300 | 5 | 2 | 1 | 690 |
|  | Sugd – Sirdarya | 220 | AC300+AC400 | 5+12 | 2 | 1 | 690 |
| PLANNED POWER LINES IN THE PERSPECTIVE 2025 |  |  |  |  |  |  |  |
|  | Rogun HPP- Dushanbe | 500 | АСх400 | 100 | 2 | 3 | 2000 |
|  | Regar – Sangtuda konver st | 500 | АСх400 | 115 | 1 | 3 | 2000 |
|  | Rogun HPP - Sangtuda konver st | 500 | АСх400 | 138 | 1 | 3 | 2000 |
|  | Rogun HPP - Saihun | 500 | АСх400 | 285 | 1 | 3 | 2000 |
|  | Saihun - Sugd | 500 | АСх400 | 65 | 1 | 3 | 2000 |
|  | Rogun HPP – Rogun | 220 | АС-400 | 8 | 2 | 1 | 835 |
|  | Shar shar – Dangara | 220 | АС-400 | 24 | 1 | 1 | 835 |
|  | Dangara – Sangtuda HPP | 220 | АС-400 | 15 | 1 | 1 | 835 |
|  | Sangtuda HPP – SS-220 kV konver st | 220 | АС-400 | 12 | 1 | 1 | 835 |
|  | Aini – Rudaki | 220 | АС-300 | 95 | 1 | 1 | 600 |
|  | Kanibadam – Asht | 220 | АС-300 | 58 | 1 | 1 | 600 |